INTRODUCTION

IN recent years, autonomous driving (or robot-assisted driving) has emerged as a fast-growing research area.

The race towards fully autonomous vehicles pushed many large companies, such as Google, Toyota and Ford, to develop their own concept of robot-car [1], [2], [3]. While self-driving cars are widely considered to be a major development and testing ground for the real-world application of artificial intelligence, major reasons for concern remain in terms of safety, ethics, cost, and reliability [4]. From a safety standpoint, in particular, smart cars need to robustly interpret the behavior of the humans (drivers, pedestrians or cyclists) they share the environment with, in order to cope with their decisions. Situation awareness and the ability to understand the behavior of other road users are thus crucial for the safe deployment of autonomous vehicles (AVs).

The latest generation of robot–cars is equipped with a  range of different sensors (i.e., laser rangefinders, radar, cameras, GPS) to provide data on what is happening on the road [5]. The information so extracted is then fused to suggest how the vehicle should move [6], [7], [8], [9]. Some authors, however, maintain that vision is a sufficient sense for AVs to navigate their environment, supported by humans’ ability to do just so. Without enlisting ourselves as supporters of the latter point of view, in this paper we consider the context of vision-based autonomous driving [10] from video sequences captured by cameras mounted on the vehicle in a streaming, online fashion.

While detector networks [11] are routinely trained to facilitate object and actor recognition in road scenes, this simply allows the vehicle to ’see’ what is around it. The philosophy of this work is that robust self-driving capabilities require a deeper, more human-like understanding of dynamic road environments (and of the evolving behavior of other road users over time) in the form of semantically meaningful concepts, as a stepping stone for intention prediction and automated decision making. One advantage of this approach is that it allows the autonomous vehicle to focus on a much smaller amount of relevant information

when learning how to make its decisions, in a way arguably

closer to how decision making takes place in humans.

On the opposite side of the spectrum lies end-to-end reinforcement learning. There, the behavior of a human driver in response to road situations is used to train, in an imitation learning setting [12], an autonomous car to respond in a more ‘human-like’ manner to road scenarios. This, however, requires an astonishing amount of data from a myriad of road situations. For highway driving only, a relatively simple task when compared to city driving, Friedman et al. in [13] had to use a whole fleet of vehicles to collect 45 million frames. Perhaps more importantly, in this approach the network learns a mapping from the scene to control inputs, without attempting to model the significant facts taking place in the scene or the reasoning of the agents therein. As discussed in [14], many authors [15], [16] have recently highlighted the insufficiency of models which directly map observations to actions [17], specifically in the self-driving cars scenario.

1.1 ROAD: A Multi-Label, Multi-Task Dataset Concept. This work aims to propose a new framework for situation awareness and perception, departing from the disorganized collection of object detection, semantic segmentation or pedestrian intention tasks which is the focus of much current work. We propose to do so in a “holistic”, multi-label approach in which agents, actions and their locations are all ingredients in the fundamental concept of road event (RE). Road events are defined as triplets E ¼ ðAg; Ac; LocÞ composed by an active road agent Ag, the action(s) Ac it performs (possibly more than one at the same time), and the location(s) Loc in which this takes place (which may vary from the start to the end of the event itself), as seen from the point of the view of an autonomous vehicle.

This takes the problem to a higher conceptual level, in which AVs are tested on their understanding of what is going on in a dynamic scene rather than their ability to describe what the scene looks like, putting them in a position to use that information to make decisions and a plot course of action. Modeling dynamic road scenes in terms of road events can also allow us to model the causal relationships between what happens; these causality links can then be exploited to predict further future consequences.

To transfer this conceptual paradigm into practice, this paper introduces ROAD, the first Road event Awareness in Autonomous Driving Dataset, as an entirely new type of dataset designed to allow researchers in autonomous vehicles to test the situation awareness capabilities of their stacks in a manner impossible until now. Unlike all existing benchmarks, ROAD provides ground truth for the action performed by all road agents, not just humans. In this sense ROAD is unique in the richness and sophistication of its annotation, designed to support the proposed conceptual shift. We are confident this contribution will be very useful moving forward for both the autonomous driving and the computer vision community.

Features. ROAD is built upon (a fraction of) the Oxford Robot Car Dataset [18], by carefully annotating 22 carefully selected, relatively long-duration videos. Road events are represented as ’tubes’, i.e., time series of frame-wise bounding box detections. ROAD is a dataset of significant size, most notably in terms of the richness and complexity of its annotation rather than the raw number of video frames. A total of 122K video frames are labeled for a total of 560K detection bounding boxes in turn associated with 1:7M unique individual labels, broken down into 560K agent labels, 640K action labels and 499K location labels.

The dataset was designed according to the following principles.

_ A multi-label benchmark: each road event is composed by the label of the (moving) agent responsible, the label(s) of the type of action(s) being performed, and labels describing where the action is located.

_ Each event can be assigned multiple instances of the same label type whenever relevant (e.g., an RE can be an instance of both moving away and turning left).

_ The labeling is done from the point of view of the AV: the final goal is for the autonomous vehicle to use this information to make the appropriate decisions.

_ The meta-data is intended to contain all the information required to fully describe a road scenario: an illustration of this concept is given in Fig. 1. After closing one’s eyes, the set of labels associated with the current video frame should be sufficient to recreate the road situation in one’s head (or, equivalently, sufficient for the AV to be able to make a decision).

In an effort to take action detection into the real world, ROAD moves away from human body actions almost entirely, to consider (besides pedestrian behavior) actions performed by humans as drivers of various types of vehicles, shifting the paradigm from actions performed by human bodies to events caused by agents. As shown in our experiments, ROAD is more challenging than current action detection benchmarks due to the complexity of road events happening in real, non-choreographed driving conditions, the number of active agents present and the variety of weather conditions encompassed.

Tasks. ROAD allows one to validate manifold tasks associated with situation awareness for self-driving, each associated with a label type (agent, action, location) or combination thereof: spatiotemporal (i) agent detection, (ii) action detection, (iii) location detection, (iv) agent-action detection, (v) road event detection, as well as the (vi) temporal segmentation of AV actions. For each task one can assess both frame-level detection, which outputs independently for each video frame the bounding box(es) (BBs) of the instances there present and the relevant class labels, and video-level detection, which consists in regressing the whole series of temporally-linked bounding boxes (i.e., in current terminology, a ’tube’) associated with an instance, together with the relevant class label. In this paper we conduct tests on both. All tasks come with both the necessary annotation and a shared baseline, which is described in Section 4.

1.2 Contributions The major contributions of the paper are thus the following.

_ A conceptual shift in situation awareness centered on a formal definition of the notion of road event, as a triplet composed by a road agent, the action(s) it performs and the location(s) of the event, seen from the point of view of the AV.

_ A new Road event Awareness Dataset for Autonomous Driving (ROAD), the first of its kind, designed to support this paradigm shift and allow the testing of a range of tasks related to situation awareness for autonomous driving: agent and/or action detection, event detection, ego-action classification.

Instrumental to the introduction of ROAD as the benchmark of choice for semantic situation awareness, we propose a robust baseline for online action/agent/event detection (termed 3D-RetinaNet) which combines state-of the- art single-stage object detector technology with an online tube construction method [19], with the aim of linking detections over time to create event tubes [20], [21]. Results for two additional baselines based on a Slow fast detector architecture [22] and YOLOv51 (for agent detection only) are also reported and critically assessed. We are confident that this work will lay the foundations upon which much further research in this area can be built.

1.3 Outline The remainder of the paper is organised as follows. Section 2 reviews related work concerning existing datasets, both for autonomous driving (Section 2.1) and action detection (Section 2.2), as well as action detection methods (Section 2.3).Section 3 presents our ROAD dataset in full detail, including: its multi-label nature (Section 3.1), data collection (Section 3.2), annotation (Section 3.3), the tasks it is designed to validate (Section 3.4), and a quantitative summary (Section 3.5). Section 4 presents an overview of the proposed 3D-RetinaNet baseline, and recalls the ROAD challenge organized by some of us at ICCV 2021 to disseminate this new approach to situation awareness within the autonomous driving and computer vision communities, using ROAD as the benchmark. Experiments are described in Section 5, where a number of ablation studies are reported and critically analyzed in detail, together with the results of the ROAD challenge’s top participants. Section 6 outlines additional exciting tasks the dataset can be used as a benchmark for in the near future, such as future event anticipation, decision making and machine theory of mind [14]. Conclusions and future work are outlined in Section 7.

The Supplementary material, which can be found on the Computer Society DigitalLibraryathttp://doi.ieeecomputersociety.org/10.1109/TPAMI.2022.3150906, reports detailed class-wise results, a qualitative analysis of success and failure cases, and a link to a 30-minute footage visually illustrating the baseline’s predictions versus the ground truth.