ROAD The Road Event Awareness Dataset for Autonomous Driving

ABSTRACT

Humans drive in a holistic fashion which entails, in particular, understanding dynamic road events and their evolution. Injecting these capabilities in autonomous vehicles can thus take situational awareness and decision making closer to human-level performance. To this purpose, we introduce the ROad event Awareness Dataset (ROAD) for Autonomous Driving, to our knowledge the first of its kind. ROAD is designed to test an autonomous vehicle’s ability to detect road events, defined as triplets composed by an active agent, the action(s) it performs and the corresponding scene locations. ROAD comprises videos originally from the Oxford RobotCar Dataset, annotated with bounding boxes showing the location in the image plane of each road event. We benchmark various detection tasks, proposing as a baseline a new incremental algorithm for online road event awareness termed 3D-RetinaNet.

We also report the performance on the ROAD tasks of Slowfast and YOLOv5 detectors, as well as that of the winners of the ICCV2021 ROAD challenge, which highlight the challenges faced by situation awareness in autonomous driving. ROAD is designed to allow scholars to investigate exciting tasks such as complex (road) activity detection, future event anticipation and continual learning.

EXISTING SYSTEM

Single-Modality Datasets. Collecting and annotating RGB data only is relatively less time-consuming and expensive than building multimodal datasets including range data from LiDAR or radar. Most single-modality datasets [23], [24], [25], [26], [27], [28] provide 2D bounding box and scene segmentation labels for RGB images. Examples include Cityscapes [24], Mapillary Vistas [25], BDD100k [26] and Apolloscape [27]. To allow the studying of how vision algorithms generalise to different unseen data, [25], [26], [28] collect RGB images under different illumination and weather conditions.

Other datasets only provide pedestrian detection annotation [29], [30], [31], [32], [33], [34], [35]. Recently, MIT and Toyota have released DriveSeg, which comes with pixellevel semantic labelling for 12 agent classes [36]. Multimodal Datasets. KITTI [37] was the first-ever multimodal dataset. It provides depth labels from front-facing stereo images and dense point clouds from LiDAR alongside GPS/IMU (inertial) data. It also provides bounding- box annotations to facilitate improvements in 3D object detection. H3D [38] and KAIST [39] are two more examples of multimodal datasets. H3D provides 3D box annotations, using real-world LiDAR-generated 3D coordinates, in crowded scenes.

Unlike KITTI, H3D comes with object detection annotations in a full 360_ view. KAIST provides thermal camera data alongside RGB, stereo, GPS/IMU and LiDAR-based range data. Among other notable multimodal datasets [18], [40] only consist of raw data without semantic labels, whereas [41] and [42] provide labels for location category and driving behaviour, respectively. The most recent multimodal large-scale AV datasets [43], [44], [45], [46], [47], [48] are significantly larger in terms of both data (also captured under varying weather conditions, e.g., by night or in

the rain) and annotations (RGB, LiDAR/radar, 3D boxes). For instance, Argovers [43] doubles the number of sensors in comparison to KITTI [37] and nuScenes [49], providing 3D bounding boxes with tracking information for 15 objects of interest. Similarly, Lyft [44] provides 3D bounding boxes for cars and location annotation including lane segments, pedestrian crosswalks, stop signs, parking zones, speed bumps, and speed humps. In a setup similar to KITTI’s [37], in KITTI-360 [48] two fisheye cameras and a pushbroom laser scanner are added to have a full 360_ field of view.

KITTI-360 contains semantic and instance annotations for both 3D point clouds and 2D images, which include 19 objects. IMU/GPS sensors are added for localisation purposes. Both 3D bounding boxes based on LiDAR data and 2D annotation on camera data for 4 objects classes are provided in Waymo [45]. In [46], using similar 3D annotation for 5 objects classes, the authors provide a more challenging dataset by adding more night-time scenarios using a faster moving car. Amongst large-scale multimodal datasets, nuScenes [49], Lyft L5 [44], Waymo Open [45] and A*3D [46] are the most dominant ones in terms of number of instances, the use of high-quality sensors with different types of data (e.g., point clouds or 360_ RGB videos), and richness of the annotation providing both semantic information and 3D bounding boxes. Furthermore, nuScenes [49], Argoverse [43] Lyft L5 [44] and KITTI-360 [48] provide contextual knowledge through human-annotated rich semantic maps, an important prior for scene understanding.

Trajectory Prediction. Another line of work considers the problem of pedestrian trajectory prediction in the autonomous driving setting, and rests on several influential RGB based datasets. To compile these datasets, RGB data were captured using either stationary surveillance cameras [50], [51], [52] or drone-mounted ones [53] for aerial view. [54], [55] use RGB images capturing an egocentric view from a moving car for future trajectory forecasting. Recently, the multimodal 3D point cloud-based datasets [37], [38], [43], [44], [45], [49], initially introduced for the benchmarking of 3D object detection and tracking, have been taken up for trajectory prediction as well. A host of interesting recent papers [56], [57], [58], [59] do propose datasets to study the intentions and actions of agents using cameras mounted on vehicles. However, they encompass a limited set of action labels (e.g., walking, standing, looking or crossing), wholly insufficient for a thorough study of road agent behaviour. Among them, TITAN [59] is arguably the most promising.

Disadvantages

• The complexity of data: Most of the existing machine learning models must be able to accurately interpret large and complex datasets to road events.

• Data availability: Most machine learning models require large amounts of data to create accurate predictions. If data is unavailable in sufficient quantities, then model accuracy may suffer.

• Incorrect labeling: The existing machine learning models are only as accurate as the data trained using the input dataset. If the data has been incorrectly labeled, the model cannot make accurate predictions.

Proposed System

A conceptual shift in situation awareness centred on a formal definition of the notion of road event, as a triplet composed by a road agent, the action(s) it performs and the location(s) of the event, seen from the point of view of the AV.

A new ROad event Awareness Dataset for Autonomous Driving (ROAD), the first of its kind, designed to support this paradigm shift and allow the testing

of a range of tasks related to situation awareness for autonomous driving: agent and/or action detection, event detection, ego-action classification.

This work aims to propose a new framework for situation awareness and perception, departing from the disorganized collection of object detection, semantic segmentation or pedestrian intention tasks which is the focus of much current work. We propose to do so in a “holistic”, multi-label approach in which agents, actions and their locations are all ingredients in the fundamental concept of road event (RE).

This takes the problem to a higher conceptual level, in which AVs are tested on their understanding of what is going on in a dynamic scene rather than their ability to describe what the scene looks like, putting them in a position to use that information to make decisions and a plot course of action. Modeling dynamic road scenes in terms of road events can also allow us to model the causal relationships between what happens; these causality links can then be exploited to predict further future consequences.

To transfer this conceptual paradigm into practice, this paper introduces ROAD, the first ROad event Awareness in Autonomous Driving Dataset, as an entirely new type of dataset designed to allow researchers in autonomous vehicles to test the situation awareness capabilities of their stacks in a manner impossible until now. Unlike all existing benchmarks, ROAD provides ground truth for the action performed by all road agents, not just humans. In this sense ROAD is unique in the richness and sophistication of its annotation, designed to support the proposed conceptual shift. We are confident this contribution will be very useful moving forward for both the autonomous driving and the computer vision community.

Advantages

_ A multi-label benchmark: each road event is composed by the label of the (moving) agent responsible, the label(s) of the type of action(s) being performed, and labels describing where the action is located.

_ Each event can be assigned multiple instances of the same label type whenever relevant (e.g., an RE can be an instance of both moving away and turning left).

_ The labeling is done from the point of view of the AV: the final goal is for the autonomous vehicle to use this information to make the appropriate decisions.

_ The meta-data is intended to contain all the information required to fully describe a road scenario: an illustration of this concept is given in this system. After closing one’s eyes, the set of labels associated with the current video frame should be sufficient to recreate the road situation in one’s head (or, equivalently, sufficient for the AV to be able to make a decision).

SYSTEM REQUIREMENTS

➢   H/W System Configuration:-

➢    Processor                      -   Pentium –IV

➢   RAM                              - 4 GB (min)

➢   Hard Disk                      -   20 GB

➢   Key Board                     -    Standard Windows Keyboard

➢   Mouse                            -    Two or Three Button Mouse

➢   Monitor                          -   SVGA

SOFTWARE REQUIREMENTS:

- Operating system 	:   Windows 7 Ultimate.

- Coding Language		:   Python.

- Front-End			:   Python.

- Back-End			:   Django-ORM

- Designing			:   Html, css, javascript.

- Data Base			:   MySQL (WAMP Server).